Traveling Waves and Phasor Homework (30 Points)

(jas, Traveling Waves and Phasor HW.docx, 9/20/2025)

Use units and clearly label answers using 3 or 4 significant digits where appropriate. Show your work so that if necessary partial credit can be awarded.

- Determine the wavelength in meters of a 2 GHz electromagnetic wave traveling through a vacuum, meaning up = c. (2 points.)

- Determine the wavelength in miles of a 60 Hz electromagnetic wave traveling along a power line at a speed of 70% of the speed of light. (2 points.)

- Two waves y1(t) and y2(t) have the same amplitude and frequency, but y2(t) leads y1(t) by 60°.  Given y1(t) = cos[2π(1000)t], write the expression for y2(t) in terms of the cosine function using radians for the argument. (3 points.)

- The voltage of an electromagnetic wave traveling on a lossy transmission line is given as follows where z is the distance in meters from the source:

vz,t=5e-αzsin4π x 109t-20πz.

- Find the frequency in Hz, wavelength in meters (m) and phase velocity of vz,t in meters per second (m/s). (2 points each for 6 points total for part a.)

- Find α given that the wave envelope equals 1.5 V for z = 2 meters. (2 points)

- Find the phasor representations of the following time functions. Simplify your results to be of the form rejθr with r being a positive quantity and θr in radians.  (15 points total.)

- v(t) = 9cos(ωt – π/3) V.  (2 points.)

- v(t) = 12sin(ωt + π/4) V.  (2 points.)

- i(x, t) = 5e-3xsin(ωt – βx + π/6) A.  (3 points.)

- i(t) = -2cos(ωt + 3π/4) A.  (4 points.)

- i(t) = 4sin(ωt + π/3) + 3cos(ωt - π/6) A.  (4 points.)